Introduction

Hey, I'm Robby!

If you're reading this, chances are we met at SAINTCON 2026, and you walked away with one of my minibadges!

A little background: this is my first attempt at designing and building a minibadge, so if you notice some rookie design decisions along the way, well... now you know why!

I'm relatively new to electronics and PCB design, so this project was a learning experience from start to finish. With plenty of help from ChatGPT, I went from knowing virtually nothing about KiCad or microcontrollers to designing, assembling, and programming my first custom PCB.

And honestly? I had a blast doing it! It's already inspired a few more projects, including building an attenuator for amateur radio direction finding (ARDF), better known as fox hunting.

That hands-on approach to learning is also what inspired ZeroStack Labs, a project I started with my buddy Alex. We document our experiments, successes, and occasional spectacular failures as we explore cybersecurity, electronics, home labs, and whatever else catches our interest.

Our philosophy is simple: Build | Break | Learn | Repeat.

If you'd rather watch the assembly process than read through these instructions, I've included a link to a video below. Fair warning: it's about 30 minutes of me figuring things out as I go! 😂

Thanks for checking out my first minibadge, and I hope you enjoy putting it together!

>_ Robby
>_ TH3R0BH0G

Link to Project Repository:
https://github.com/ZeroStack-Labs/saintcon-2026-minibadge

Contact Me:

Email: kj6gcs@gmail.com

LinkedIn: https://www.linkedin.com/in/robbywideman

GitHub (Personal): https://www.github.com/kj6gcs

GitHub (ZeroStack Labs): https://www.github.com/ZeroStack-Labs

ZeroStack Labs Website: https://www.zerostacklabs.dev

Personal Portfolio Website: https://www.robbywideman.dev

YouTube: https://www.youtube.com/@zerostacklabs10

Bill of Materials:

1 x Minibadge PCB

(Custom order through OSH Park (https://www.oshpark.com)

4 x 2 Pin Male Header Pins

(https://www.digikey.com/en/products/detail/sullins-connector-solutions/PREC040SAAN-RC/2774814)

1 x Preprogrammed ATtiny402-SSN MCU (8BIT 4KB Flash 8SOIC)

(https://www.digikey.com/en/products/detail/microchip-technology/ATTINY402-SSN/9947535)

2 x 220Ω ±1% 0.25W, 1/4W Chip Resistor 1206 (3216 Metric)

(https://www.digikey.com/en/products/detail/yageo/RC1206FR-07220RL/728693)

2 x Green 570nm Green LEDs - Discrete 2.1V 1206 (3216 Metric)

(https://www.digikey.com/en/products/detail/kingbright/APTL3216CGCK/1747561)

1 x 0.1 µF ±10% 10V Ceramic Capacitor X7R 0805 (2012 Metric)

(https://www.digikey.com/en/products/detail/kemet/C0805C104K8RACTU/2211745)

3 x 2M*4L Heat Inserts

(https://www.amazon.com/dp/B0DK16VY81)

3 x 2M*6L Screws

(https://www.amazon.com/dp/B0H5K78SPY)

1 x Soldering Jig to hold your board/pins still during assembly

Custom Made / 3D Printed by me! (Printables.com) (GitHub)

Badge Topper Variations

Custom Made / 3D Printed by me! (Printables.com) (GitHub)